The study focuses on minimizing fuel consumption and greenhouse gas (GHG) emissions of cargo vessels ( tankers, bulk carriers , containers etc..). The main contributor is the vessel speed:
There is a nonlinear relationship between the SWS ( Still water speed ) - the speed that the engine produces, and the overall fuel consumption. It means that small changes in engine speed result in an increase in the power ( between 2.7 and 3.3 ). If the only target during a vessel voyage is to reduce fuel consumption, the answer will be to sail at the lowest speed possible.  Adhering to desired time of arrival (DTA), is the place where a vessel must increase engine speed, and consume additional fuel. Another consideration of speed control, is the actual SOG ( speed over ground ), this is the linear speed that combines the speed gained from the engine ( SWS ) and weather and sea conditions ( wind, wave , currents .. ) . Using physical formulas that translate weather and sea conditions, combined with naval architecture standards, the calculation of speed consumption and SOG, in the case of different engine power (SWS) is fairly feasible.

As a graduate student in machine learning and operation research, I want this thesis to contribute to the following question:
For a fixed ship route divided into n segments between departure port A and destination port

B where the bearing of the ship in each segment is fixed, determine the optimal Still

Water Speed (SWS) for each segment to minimize the total fuel consumption over the

voyage while ensuring arrival at port B is not later than a desired time of arrival (DTA).

Each segment has fixed and known weather and sea conditions (wind, waves, and ocean

currents) affecting the vessel’s speed.

First step of the research is to proove that by modeling the problem as a linear optimization problem, can produce an optimal solution, superior to a heuristic approach.
The benchmark for heuristic solution, is a journal  article named “ Ship Speed Optimization Considering Ocean Currents to Enhance Environmental Sustainability in Maritime Shipping “. the article provides 2 main important factors:

- Detailed formulation of maritime parameters calculation ( SOG, Fuel consumption..)

- Detailed use case of a 12 segment voyage , with weather and sea condition data

The article provides the solution approach, and the minimal fuel consumption that the algorithm achieved.
The first part of the thesis \ journal article, is to provide:

- Literature review

- Research approach

- Problem definition and mathematical formulation

- Results and comparison against heuristic approach baseline

In order to "level the playing field” , I've first implemented all the equations from the article, into a python code. The target was to use the same baseline of parameters calculations ( SOG, FCR) in order to create a valid .DATA file. The file with all the necessary calculation, was then used in a linear programing model, that produced a new set of optimised results.

The first step of validation was to take the article’s SWS input, SOG output, and compare it with the results of the same inputs through my utility functions implementation. The goal was to see minimum discrepancies between article SOG and calculated SOG. results below:

| Segment | SWS | Article SOG | Calculated Sog | sog diff | percent diff |
| --- | --- | --- | --- | --- | --- |
| 1 | 12.7 | 12.36 | 12.356215 | 0.003785 | 0.03% |
| 2 | 12.2 | 11.72 | 11.738374 | 0.018374 | 0.16% |
| 3 | 12.2 | 12.59 | 12.635041 | 0.045041 | 0.36% |
| 4 | 12.1 | 12.11 | 12.132424 | 0.022424 | 0.19% |
| 5 | 12.5 | 12.04 | 12.03888 | 0.00112 | 0.01% |
| 6 | 12.3 | 12.1 | 12.165201 | 0.065201 | 0.54% |
| 7 | 12.4 | 11.85 | 11.852915 | 0.002915 | 0.02% |
| 8 | 12.7 | 10.98 | 10.809994 | 0.170006 | 1.55% |
| 9 | 12.3 | 12.05 | 12.007468 | 0.042532 | 0.35% |
| 10 | 12 | 12.67 | 12.660112 | 0.009888 | 0.08% |
| 11 | 12.4 | 12.21 | 12.222302 | 0.012302 | 0.10% |
| 12 | 12.5 | 12.72 | 12.672564 | 0.047436 | 0.37% |

The next step was to compare optimization results between the article Genetic algorithm and linear programing method. Results below:

Optimization results - genetic algorithm ( Article ):

| Segment | SWS | SOG | Travel_Time | Distance | FCR | Fuel_Consumption |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 12.7 | 12.36 | 18.1 | 223.716 | 1.44 | 26.06 |  |
| 2 | 12.2 | 11.72 | 24.1 | 282.452 | 1.29 | 31.09 |  |
| 3 | 12.2 | 12.59 | 24.1 | 303.419 | 1.29 | 31.09 |  |
| 4 | 12.1 | 12.11 | 24.6 | 297.906 | 1.25 | 30.75 |  |
| 5 | 12.5 | 12.04 | 23.3 | 280.532 | 1.38 | 32.15 |  |
| 6 | 12.3 | 12.1 | 23.8 | 287.98 | 1.32 | 31.42 |  |
| 7 | 12.4 | 11.85 | 24 | 284.4 | 1.35 | 32.4 |  |
| 8 | 12.7 | 10.98 | 21.2 | 232.776 | 1.44 | 30.53 |  |
| 9 | 12.3 | 12.05 | 25.1 | 302.455 | 1.32 | 33.13 |  |
| 10 | 12 | 12.67 | 24.9 | 315.483 | 1.21 | 30.13 |  |
| 11 | 12.4 | 12.21 | 24.1 | 294.261 | 1.35 | 32.54 |  |
| 12 | 12.5 | 12.72 | 22.7 | 288.744 | 1.38 | 31.33 |  |
|  |  |  | 280 | 3394.124 |  | 372.62 |  |

Optimization results - Linear programing :

| Segment | SWS | SOG | Travel_Time | Distance | FCR | Fuel_Consumption |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 12.5 | 12.154758 | 18.41747898 | 223.86 | 1.38 | 25.40 |  |
| 2 | 12.4 | 11.939477 | 23.66435314 | 282.54 | 1.35 | 31.85 |  |
| 3 | 12 | 12.431751 | 24.38755409 | 303.18 | 1.22 | 29.75 |  |
| 4 | 12.2 | 12.234161 | 24.39398991 | 298.44 | 1.28 | 31.27 |  |
| 5 | 12.6 | 12.143079 | 23.10040147 | 280.51 | 1.41 | 32.62 |  |
| 6 | 12.5 | 12.374831 | 23.21971104 | 287.34 | 1.38 | 32.02 |  |
| 7 | 12.5 | 11.953601 | 23.79199373 | 284.4 | 1.38 | 32.81 |  |
| 8 | 13.2 | 11.313922 | 20.61619304 | 233.25 | 1.62 | 33.48 |  |
| 9 | 12.3 | 12.007468 | 25.13435805 | 301.8 | 1.31 | 33.02 |  |
| 10 | 11.8 | 12.45893 | 25.33925466 | 315.7 | 1.16 | 29.39 |  |
| 11 | 12.3 | 12.121203 | 24.23851824 | 293.8 | 1.31 | 31.84 |  |
| 12 | 12 | 12.171735 | 23.6958823 | 288.42 | 1.22 | 28.91 |  |
|  |  |  | 279.9996887 | 3393.24 |  | 372.37 |  |